About these 1:250k GIS data sets			Charlie Richman 12/10/01

Introduction

These GIS data sets were produced in the course of analyzing digital elevation information for coastal areas of the contiguous 48 states.  This broad-area study used 1:250,000 scale Digital Elevation Model information from the USGS as the primary data source.  Explicit shoreline information was added from the NOAA Medium Resolution Vector shoreline.  Manual edits were added in some areas to correct obvious errors in the source DEM data.  These data sets provide the core information content of the map series generated from the 1:250,000 scale DEM data.

Data Format

These data sets are provided in the ESRI ArcInfo coverage format used in the original study.  Each data set corresponds to one of the 1 degree by 1 degree source data sets provided by USGS.  See http://edcwww.cr.usgs.gov/doc/edchome/ndcdb/ndcdb.html for important information on the characteristics of the source data sets.  Specifically the following URL will lead to an index map of 1:250 scale USGS quadrangle maps that correlate by name with the elevation data files from the GIS Data directory on the CDROM. See http://edcwww.cr.usgs.gov/glis/hyper/guide/1_dgr_demfig/index1m.html.

The data sets provided on this disk differ from the source data sets in several important respects:

- The source data sets provide elevation values for lattice points spaced 3 arc-seconds apart.  Elevation values are provided in units of whole (integer) meters above the National Geodetic Vertical Datum of 1929.  The data sets on this disk are vector polygons.  Each polygon has a single elevation value, and was derived from the original DEM elevation values using the ArcInfo DEMLATTICE and LATTICEPOLY commands..

- The current data sets combine all values equal to or greater than 4 meters above NGVD into a single elevation class, minimizing the number of polygons to be stored.

- The source data were provided in geographic (latitude/longitude) units. The data sets on this disk have been converted to Albers projected meters units.  (Parameters: central meridian: -96, standard parallels 29.5 and 45.5 degrees, latitude of origin: 23, North American Datum 1983.)  Coordinate conversion was performed after conversion to vector format to minimize spatial error.

- There is no explicit shoreline in the DEM source data (a value of 0 can either mean open water or land less than 0.5 m above NGVD.)  NOAA Medium Resolution Vector Shoreline data were used to distinguish areas representing water from areas representing land.  See http://spo.nos.noaa.gov/projects/shoreline/shoreline.html.  Note NOAA provides purely linear data; we created polygons by connecting the NOAA arcs to create polygons.  We also tagged those polygons as land or water as appropriate.  In some cases we may have missed small coastal lakes and coded those areas as land.  We also split very large and intricate polygons into smaller units as necessary to accommodate internal limitations of the ArcInfo 7.2 and 8.0 software.

- We identified several areas where errors inherent in the source DEM data were objectionable for the purposes of our study.  These data sets include both edited and raw elevation values.  Only selected areas were reviewed and updated.  None of the west coast data sets have been reviewed in this fashion.

Polygon attributes:

Area

ArcInfo generated value in square meters

Perimeter

ArcInfo generated value in meters

internal ID

ArcInfo generated internal identifier

user ID

ArcInfo generated user identifier

range-code

Derived from the elevation values supplied in the source DEM.  Like the original source, provided in units of whole meters.  Unlike the original source, values higher than 4 are coded as 4.  Our maps generally do not use this data field directly (see jim_elev.)  For those that do, land polygons with values coded 0 or 1 are shown in red and interpreted as up to 1.5 meters above NGVD, land polygons with values coded 2 or 3 are coded blue and interpreted as up to 3.5 meters above NGVD, and land polygons with the value 4 are shown in light yellow and interpreted as uplands.  We disregard this data field for polygons that appear to represent water.

jim_elev

This field reflects both the source DEM information and limited manual edits to resolve apparent errors based on review of selected USGS 7.5 minute series paper maps.  West coast areas were not reviewed in this fashion.  Suspect areas were hand-sketched on small-format index sheets and digitized; the digitized data were used to create an updated copy of the same information contained in the range-code field.  Note that the manual process distinguished between areas to be coded red, blue, or light yellow – not between individual integer values.  Although the values 0 through 4 may occur, there is no meaningful distinction in this data field between the values 0 and 1 or between the values 2 and 3. For polygons that represent land, 0 or 1 are interpreted as areas up to 1.5m above NGVD, values 2 or 3 are interpreted as areas up to 3.5m above NGVD, and the value 4 represents upland areas.  We disregard these values for polygons that appear to represent water.  Note that this editing process was designed as a rapid way to exclude areas from appearing to be very low-lying areas based on the source DEM data when we have other information clarifying that they are not actually so close to sea level.  No attempt was made to adhere to any particular spatial accuracy standard.  Users are cautioned that these edits were intended to be rendered on small scale maps only; most of our maps were produced at a scale of 1:1,000,000 or more.  Edits typically extend well into the ocean because our process relied on the NOAA shoreline to draw that boundary.

edit_elev

The higher of range-code and jim_elev.  Not used.

titus_spline#

Not used.

titus_spline-id

Not used.

Class

Not used directly, but reflects the information used to create the updated jim_elev field from the un-edited range-code field.  For polygons that represent land, R=red (up to 1.5m above NGVD), B=blue (up to 3.5m above NGVD), N=neither(uplands 3.5m or more above NGVD).  This data field was the result of intersecting a separate ArcInfo coverage containing edits with each individual data set as appropriate.

ts3c_bd#

Not used.

ts3c_bd-id

Not used.

class_v2

Not used.

class_v1

Not used.

noaa-water

Distinguishes land from water based on the NOAA Medium Resolution Vector shoreline.  “y” = water (printed light blue on our maps), “n” = land (printed according to the contents of jim_elev on our maps.)  Note that our production process involved converting the linear shoreline provided by NOAA to polygons and coding those polygons as land and water as appropriate.  In some cases we may have missed inland ponds and falsely coded those areas as land.

stateabb

2-digit state abbreviations, in upper case, used for statistical tabulations.

Additional notes on these data sets:

- These specific data sets were derived from a series of other data products used for a variety of cartographic and analytic purposes.  They contain several data fields whose values are no longer useful.

- These specific data sets were assembled to support statistical analysis of elevation information we had already mapped.  They were created after the associated mapping effort was complete.  The mapping effort itself did not rely on elevation and shoreline data combined into a single data set; blue open water areas were simply drawn on top of the elevation data. The information contained in the data sets on this disk is substantially identical to the information shown in our maps.  These are the cartographic differences between the maps we created and the data sets on this disk:

    - Areas in Canada and Mexico are obscured on our maps.

    - Inland areas not included in these data sets are added and drawn as uplands (3.5 meters or more above NGVD) on our maps.

    - Ocean areas not included in our data sets are added and shown as open water.

    - Major lakes, including the Great Lakes and Lake Okeechobee, are shown on our maps.

    - Cities, Highways, State and County boundaries are included on some of our maps.

    - Some of our maps (e.g., for Maryland) obscure out-of-state areas.

    - DEM data in this series were not available for a small portion of North Carolina at Cape Fear.  To be conservative, lands east of 78 degrees longitude and south of 34 degrees latitude are shown as uplands (3.5 meters or more above NGVD) on our maps.

How to use these data sets in ArcMap  (ArcView, ArcEditor, or ArcInfo 8.x):

Consult the index images in the documentation directory to determine which data sets cover your area of interest, and add them to your map.

- For each data set, right-click on the data set name in the legend area, choose “properties” and the “symbology” tab.  Select Categories and “Unique Values, Many Fields”.

- Choose the fields “JIM_ELEV” and “NOAA-WATER”, and click the button to Add All Values.

- Select all of the symbols where NOAA-WATER = y, right-click, and group them together.  Those areas represent open water.  Choose an outline color of “no color.”

- Select all of the remaining symbols where JIM_ELEV is equal to zero or 1, right-click, and group them together.  Those areas represent lands below 1.5m above NGVD.  Choose an outline color of “no color.”

- Select all of the remaining symbols where JIM_ELEV is equal to 2 or 3, right-click, and group them together.  Those areas represent lands below 3.5m above NGVD.  .  Choose an outline color of “no color.”

- The remaining symbol represents upland areas with higher elevations.  Choose an outline color of “no color.”

Alternatively, you may use SampleSymbology.lyr provided with the data:

- Right-click on a data layer name in the legend area, choose “properties” and the “symbology” tab.

- Click Import, and choose the open file icon to navigate to SampleSymbology.lyr; select it, and click Add.  (For subsequent layers, it is faster to choose to layer you have already symbolized.)

- Click OK twice to accept the sample symbology.

How to use these data sets with ArcView 3.x:

Consult the index images in the documentation directory to determine which data sets cover your area of interest, and add them to your map.

- Add each data layer to the map twice.

- Double-click on the upper of the two copies to edit symbology for water.

- Choose legend type “Unique Value” and use “Noaa-water” as the values field.

- For the value “y”, choose a foreground color to represent open water, and choose an outline color of <nothing>  (shown by ArcView as an X.)

- For the value “n”, choose a foreground color of <nothing> and an outline color of <nothing>.

- Apply the changes.

- Double-click on the lower of the two copies to edit symbology for land.

- Choose legend type “Unique Value” and use “Jim_elev” as the values field.

- For the values “0” and “1”, choose a foreground color to represent land below 1.5m above NGVD, and an outline color of <nothing>.

- For the values “2” and “3”, choose a foreground color to represent land between 1.5m and 3.5m above NGVD, and an outline color of <nothing>.

- For the value “4”, choose a foreground color to represent lands above 3.5m above NGVD, and an outline color of <nothing>.

- Apply the changes.